1. Connect the power supply and connect the USB：

2. Install the driver ( software --> Driver --> CH340SER.exe ) :

3. To Determine your Machine's COM port:

- Windows XP: Right click on "My Computer", select "Properties", select "Device Manager".

- Windows 7: Click "Start" -> Right click "Computer" -> Select "Manage" -> Select "Device Manager" from left pane

- In the tree, expand "Ports (COM & LPT)"

- Your machine will be the USB Serial Port (COMX), where the “X” represents the COM number, for example COM6.

- If there are multiple USB serial ports, right click each one and check the manufacturer, the machine will be "CH340".

4. Open grblControl software(software -> Grblcontrol -> grblControl.exe)

- Console window print ” [CTRL+X] < Grbl 0.9j ['$' for help]” If the connection is successful.

- Console window print ” Serial port error 1: No such file or directory “ indicate that the connection is failed.

- Grblcontrol Use

- Tool setting